ESE 562 Project 1

Po Hsu Chen, 448031

1.

Vdd =3V

| Ib | Gain(db) | Dynamic Range | Power (3Ib*Vdd) |
| --- | --- | --- | --- |
| 1 na | 90.66 | 2.9999177432 | 9 nw |
| 10 na | 90.3 | 2.9999544002 | 90 nw |
| 100 na | 89.92 | 2.9996431078 | 900 nw |
| 1ua | 88.25 | 2.9965689450 | 9000 nw |

2.

| Ib | C | Setting Time (sec) | Overshoot |
| --- | --- | --- | --- |
| 1 ua | 0.1p | 0.000000911 | 59.02800062E-3 |
| 100 na | 0.1 p | 0.000001039 | 17.04848489E-3 |
| 10 na | 0.1 p | 0.000006241 | 270.7085913E-6 |
| 1 na | 0.1 p | 0.000095723 | 335.9898122E-6 |
| 1 ua | 1p | 0.000001317 | 14.39409119E-3 |
| 100 na | 1p | 0.000002788 | 15.94468135E-3 |
| 10 na | 1p | 0.000020056 | 1.948771228E-3 |
| 1 na | 1p | 0.00018319 | 350.935458E-6 |
| 1 ua | 10p | 0.000003481 | 222.5530934E-6 |
| 100 na | 10p | 0.000015944 | 184.690184E-6 |
| 10 na | 10p | 0.00014793 | 668.9398784E-6 |
| 1 na | 10p | 0.001409787 | 416.1767558E-6 |
| 1 ua | 0p | X |  |
| 100 na | 0p | X |  |
| 10 na | 0p | X |  |
| 1 na | 0p | X |  |

Before gain decreases to 0db, phase margin is negative. So the system is unstable and ringing. We can’t measure the setting time due to the above reason.

3.

One zero

One zero

Two pole

As the following figure (magnitude plot) shows, there are two zeros and two poles.

### 20 dB/decade drop of the pole is arrested by the 20 dB/decade rise of the zero resulting in a horizontal magnitude plot for frequencies above the zero location. As a result, the result above perfectly match the circuit. My experiment is successful.